ARPRO

Float Data Processing Software

Developed by Vassilis Zervakis

and Mike Sparrow

Introduction

ARPRO is a series of Matlab subroutines for visualising and analysing float data as well as producing a variety of statistical parameters. The software was designed to be run in conjunction with the ARTRK and ARTOA programmes designed by Martin Menzel.

ARPRO allows the processing of float data in two ways :

- Examining individual floats

- An analysis of a group of floats

ARPRO should be set up according to the instructions in the ‘readme.txt’ file supplied with the package.  You must remember to set the “PATHS” option before processing can start.

Notes to new versions :

Version 3.1 has two additional features -  (a) "PATHS" has been replaced with "SETTINGS", which gives either the "PATHS" option as before or "SET FLOAT DATA FREQ" which allows the user to specify how many position fixes per day are in the float data. (b) "Specify T" in the "Surface Statistics" option. This  can be useful if, for example, you wish to find the error of the velocities of one float but using a previously calculated value of T.

Version 3.2 : The map selecting options have been made a little easier by the addition of clickable buttons, allowing the user to easily select the regions of interest. One should still input the minimum and maximum longitudes and latitudes to zoom in on smaller regions. ARPRO should now be fully compatible with data from the Woods Hole data archive (http://wfdac.whoi.edu), though because of the conversion required its not as fast as with the rfc files. Two things to note : I have not yet had time to fully check this, so please report any bugs ; ARPRO can handle mixing different data types, but for the statistics the data you wish to use should be in the same directory.

Examining individual floats

Loading a float :

To load an individual float click on the “FILE” option and then “Open”. Data with velocities greater than 998 cm s-1 are automatically rejected - this can sometimes occur at the beginning and end of a file.

Lowpassing data :

The data may be lowpassed (in order to minimise tidal influences) by clicking on "PROCESS" and then "Lowpass". Three types of filters are provided - Boxcar, Triangle and Hanning with filter lengths of 3-7 points, though these options can easily be changed (start at harpro.m). The variables ltemp, llat, lu, lv and  lwwww are the corresponding lowpassed parameters. All the data are made global, so it is straightforward to return to the MATLAB command window and continue processing interactively.

Examining the header :

The “HEADER” option reads and prints out the header of the current float. The format of the header file will depend on the format of the original float file.

Temperature and pressure of an individual float :

The temperature and pressure records of the float (lowpassed or raw) may be examined (option “PROCESS” followed by “T+P of Float”). This can

Figure 1 : Example of T+P of a float.

sometimes be helpful in seeing whether, for example, a particular float crosses a water mass boundary or changes depth. A particularly interesting section of the temperature (T) or pressure (P) plots can be zoomed into by making use of the left and right mouse buttons (see Figure 1).

The Rossby number :

It is sometimes useful to consider the ratios between the non-linear and Coriolis terms, a ratio that is known as the Rossby Number (Ro) :

Ro=non−lineartermCoriolisterm

ARPRO allows the user two options to examine Ro :

“ROSSBY1” : Simply plots Ro against the acceleration of the float (Figure 2a).

“ROSSBY2” : Plots the Rossby number on a polar plot. Zero degrees corresponds to acceleration parallel to the Coriolis acceleration and -180 to an acceleration opposite to that of the Coriolis (Figure 2b).

Figure 2a and 2b : Rossby number plotting on Cartesian (ROSSBY1) (top) and polar (ROSSBY2) (bottom) plots.

Summary of the current float :

A useful tool to produce a data report or to keep a record of each float processed is the “FLOAT SUMMARY” option. This plots the track of the current float (note : if the float data has been lowpassed then this is used for the summary) and its temperature and pressure along the float’s path. The user is first given the choice of including bathymetry, and then can select the map limits, bathymetric contours and markers on the floats track etc. in the “Selecting map limits” screen. The easiest method to get a float track to fill the axes is to choose approximate limits and then use the left and right mouse buttons to zoom in and out of the figure. This may also be done with the temperature and pressure axes. Note that on a p.c. the entire figure may not fit on the screen, though this does not affect the final printed figure.

Analysing a group of floats :

There are several methods of examining groups of floats using ARPRO - plotting groups of floats, creating a movie of the float trajectories or carrying out statistical analyses.

Plotting float trajectories on a map :

Choose the option “PLOT MAP”. One first has to choose the type of map required - either “Colour map” or “Black and white map” and specify whether bathymetry is required. Enter the co-ordinates of the map required and other relevant parameters in the “Selecting map limits” figure. Once the map is plotted, return to the ARPRO window and either “Overlay current float” or “Overlay Rossby vectors”. To overlay further floats, return to “Open” (in the “FILE” option) and choose “Overlay …” again (see Figure 3).

To change the characteristics of  the track plotting, one has to go to the window ‘Selecting Map Characteristics’, make the required changes, and DO NOT PRESS “PLOT” but return to the “Overlay…” option.

Producing a movie of the float trajectories :

This is one of the more interesting of ARPRO’s abilities. Being able to see the float trajectories in a movie format can be useful for demonstrations, or to simply give the user a better idea of the flow pattern.

Figure 3 : Using the “PLOT MAP” function to produce float trajectories.

Go to the option “MOVIE”. As with the map plotting option above, one has to choose the type of map required (colour or black and white) and whether bathymetry is required. Once the map has been plotted, choose the option “Overlay floats” in the same section and follow the directions. The movie is run five times, after which the user has the option of replaying.

Statistical analysis :

Several types of statistics analyses may be performed on the data. The simplest is to examine the Kinetic Energy Spectra. Choose “STATISTICS” and then “K. E. Spectra”. Both log-log (Figure 4a) and energy conserving (Figure 4b) spectra may be plotted. There is also the option of plotting rotary spectra.

The section “Surface Statistics” deals with producing parameters such as Eulerian velocities, Intergral Time and Length Scales and eddy diffusivities.

To begin with, the desired map limits, grid size etc. should be entered. Note that if the map limits are changed more than once there may be a need to return to the matlab command window to clear the global variable ‘mapfigure’ (type : clear global mapfigure). If Eulerian velocities are required (and the data density is reasonably high) a small grid size may be used. If Lagrangian statistics are required then a larger grid size should be used as the statistics need a large number of observations in each box.

Figure 4a : K.E. Spectra on a log-log plot.

Figure 4b : Energy conserving K.E. Spectra on a log-log plot.

Figure 5 : Example of Eulerian velocities produced by ARPRO.

Vectors are drawn from the centre of each grid box (use the “Grid” option to see the boxes on the figure) representing the mean Eulerian velocity (see Figure 5). The results are written to the file “errorbar.asc” (see example below) which should be examined if more detail is required. Note : if rows or columns are equal to one then the values are written on screen rather than plotted (making rows and columns = 1 is a good idea if you want Lagrangian statistics and the data density are not high).

example of part of “errorbar.asc” file :

Results of statistical analysis

-------------------------------

Lon   Lat   Umean Vmean  dU		 dV		 U*	V*   dU*   dV*

dec.  deg.  cm/s  cm/s  cm/s  cm/s  cm/s  cm/s  cm/s  cm/s

-17.50  28.00 0.54  -0.18 1.32  0.65  4.57  3.70  0.91  0.45

T11   dT11  T22	dT22   L11	  dL11	 L22   dL22

---------days---------   ------kilometers------

25.79  0.38  9.63 0.24  117.79 23.61 35.65 4.44

K11	      dK11		   K22		 dK22     R.Str.  Npoints

-------------------cm^2 s^-1------------------ (/den)

46478942.25 13185145.00 11405591.13 2053215.05 1986.34 1240.00

U* = root of the mean square of u’ (= U-Umean)

This is the output of the “errorbar.asc” for every grid box. The lon/lat give the central position of each box. R.Str. (Reynolds stress/ density) give the magnitude of the Reynolds stress. Npoints are the number of data points in each box. The rest of the terms are explained below.

About the statistics :

Arriving to statistically significant conclusions from Lagrangian measurements requires the separation of the flow into mean and perturbation components. To obtain the mean velocity Umean, Vmean (zonal, meridional components respectively) of the flow, we define the ensemble (time and space) average of the velocities of all the floats within a region of spatial scale ; minimising would give us more resolution, but would increase the error Umean of the mean velocity estimate, since the product Umeanis a constant dependent on the sampling density and eddy field properties (Davis, 1991).

In quantifying the influence of the perturbation flow u´, v´ (u´ = U-Umean) on the mean flow the determination of an eddy diffusivity coefficient (K) is required.  The method of quantifying K, assuming homogeneous, stationary turbulence, is based on the following ‘Taylor’s Theorem’ (Taylor, 1921), that relates the variance of the displacement of the flow particles to the variance of their velocities:

x(t)=2u0t(t−t)R(t)dt     (1)

where x´(t) is the displacement of the particle due to u´ and R is the autocorrelation coefficient of the perturbation velocity (Krauss & Böning, 1987).  The integral time scale T, defined by

Tº0¥R(t)dt,

defines two different limits of the integral of  Taylor’s Theorem:

- when t  0, i.e.  t < T,  then x=ut2, (initial dispersion),

- when t >>T, then x=2uTt, (random walk region).

When t>>T, the Lagrangian eddy diffusivity K=12dxdt, is given by

K=uT       (2)

Another important quantity is the integral length scale L, related to T  by L=uT (Krauss & Böning, 1987).  In the following sections (and in ARPRO) we will use the notation u* ºu.

Following Krauss and Boning (1987), the error in the Eulerian velocity can be calculated from :

Umean±2u'2Nm	(3)

Umean is the mean of the velocity averaged over time and u'2 is the corresponding standard deviation. If we have N* observations and T is the integral time scale, then we have Nm= N*/T statistically independent data. The second term in (3) gives the 95% confidence interval according to the student -t test. Note : in ARPRO if an estimate of T has not been made in a certain grid box (either by autocorrelation or by stream function) then both T11 (the meridional component) and T22 (the zonal component) are set to 1.

The best method to obtain good statistical results from ARPRO is to choose the map limits and floats required, then choose “T by autocorrelation” for each grid box and finally run “Errors”.

When estimating K, several other figures are produced. In each figure either follow the instructions or press ‘Return’ to go onto the next figure.

ARPRO is still under development. If you have any problems, suggestions or whatever then please contact us at the address below. Good luck.

Vassilis Zervakis and Mike Sparrow……………….mike.sparrow@ainco.es

AINCO-Interocean Web Site……………………http ://www.ainco.es/ainco

AINCO- Interocean

Oceanografía y Medio Ambiente

Costa Rica, 11 (1,A-26)

28016 Madrid

España

REFERENCES

Davis, R.E., 1991 : Observing the general circulation with floats. Deep-Sea Res., 38, 531-571.

Krauss, W., and C. W. Böning, 1987 : Lagrangian properties of eddy fields in the northern North Atlantic as deduced from satellite-tracked buoys. J. Marine. Res., 45, 259-291.

Taylor, G. T., 1921 : Diffusion by continuous movement. Proc. London Math. Soc. Ser. A, 20, 196-221.